CSIS2270 - Lab #2

Introduction to Home Router Configuration

Name:	 Ishaan Sharma    Student No.: 300362679

**** SCREENSHOTS ATTACHED AT LAST*************

Introduction:  This lab provides an introduction to the basic networking commands and the configurations of DHCP service and wireless service on a Home Router.

Objectives:

- To get familiar with the basic networking commands.

- To get familiar with router configurations.

Equipment Required:

- PC

- Home Router.

- Smart phone.

- Straight through network cables.

- Network Address	[_____/2]

- Start Packet Tracer (log in if asked to do so) and drag a PC, Home Router and a Smart phone to the workspace

- Connect the PC to one of the four LAN ports of the Home Router with a straight network cable.

- Click on the PC and click on Desktop menu and from the top menu bar find the IP Configuration icon and click it then click on the DHCP radio button to select it, then click the X button to close the window.

- On the Desktop menu from the top menu bar find the Command Prompt icon and click it and you will get the cmd of the PC.

- Enter the command ipconfig after C:\> prompt.

- What is the IP address of your machine?    192.168.0.1

- What is the IP address of the Default Gateway?   192.168.0.101

(Note that the Default Gateway is the router that the PC is connected to.)

- Enter the command ipconfig /all

- What is the physical address (also known as the MAC address) of the Ethernet adapter?   00E0.F7.96.50B1

B. Router Configuration – DHCP Server

In the above configuration, the router uses the Dynamic Host Configuration Protocol (DHCP) service to assign IP addresses to machines connected to the network. The network administrator can set the IP address range to be assigned or alternatively, he can turn off the DHCP server and configure IP address to each machine manually.

B.1		[_____/4]

- Logging into the Home Router

- Click on the PC and then click on Desktop. Click on the web browser icon and type the Gateway IP address in the URL box and click Go. Use “admin” user name and “admin” password and click OK. (note that if the Gateway IP is not correct you will not get the log in dialog)

- Current settings of DHCP server. From the top menu bar click Setup.

- Check the Network Setup/Router IP and DHCP Server Settings

- What is the IP address and the subnet mask of the Home Router?  255.255.255.0

- What is the start address used by the current DHCP server?   192.68.0 - In TextBox it says 100.

- What is the maximum number of users that the current DHCP server can support? - 50 Users

- How long is the client lease time (an IP address is assigned to a machine for this period of time, and if a renew request has not been received before the lease expires, the address can be assigned to another machine)?- Client lease time is zero which means one day.

- Changing the DHCP server settings

- Edit the Home Router address to 192.168.10.1 and click Save Settings at the bottom of the Setup window. What you will notice? The Home Router Setting window disappeared

Why? Because the Home Router current IP address has been changed, i can see the new IP address in the TextBox though.

- Open the PC web browser icon and type the new Home Router IP address 192.168.10.1 in the URL box and click Go to connect to the Home Router again (use the same username and password)

- Open the Command Prompt and use the command ipconfig /release to remove the IP address obtained previously.

- Use ipconfig to check if the IP address has been removed, if not, repeated the above step again.

- Use ipconfig /renew to obtain a new IP address from the Home Router.

- What is the IP address of your PC now?   192.168.10.101

B.2 Manual Configuration of IP addresses	[_____/4]

- While connected to the Home Router, disable the DHCP server on the Home Router by checking the Disabled radio button. Click Save Settings.

- In the command prompt window of the PC, use ipconfig /release to remove the IP address of the PC

- Can you still connect to the router? No I cannot

- If you cannot connect to the router, what is the reason?- Because there is no IP Address.

- Manual configuration of IP address on a PC with DHCP server disabled.

- On the PC IP Configuration window click on Static radio button

- Enter the following settings

IP address: 192.168.10.20

Subnet mask: 255.255.255.0

Default Gateway: 192.168.10.1

- From the PC Command Prompt window, use ipconfig to verify your settings.

(Actually, in the real case this is achieved by the following steps: On the PC, select Control Panel > Network and Sharing Center > Change adapter Settings > right click Local Area Connection > Properties > Networking > Internet Protocol Version 4 (TCP/IPv4) > Properties

From the new window, check Use the following IP address, and put in the required settings, then click OK.)

IP address: 192.168.10.20

Subnet mask: 255.255.255.0

Default Gateway: 192.168.10.1

B.3	[_____/4]

- Switching back to DHCP service.

- Enable the DHCP server on the Home Router (now you must know how to do it.)

- Configure the network adapter on the PC to use DHCP service by following the steps in 3 of part A above.

- Now what is the IP address of the PC?  142.179.105.85

- DHCP Reservation

- On the Home Router, under Network Setup/DHCP Server Settings, click DHCP Reservations.

- What are the devices MAC and IP addresses of the machines that are connected to the Home Router? They are- 00:90:21:65:BC:70     00:12:16:2b:B7:41

- Check the Select box of your PC and click Add Client. Your device will be assigned with the same IP address every time it connects to the network.

- Click Remove to remove the DHCP reservation.

C. Router Configuration – Wireless Network	[_____/2]

The dual-band Home Router supports both 2.4 GHz and 5 GHz bands. While you can assign the same SSID for both networks, you can also use them as two separate networks to better manage the traffic. It has been suggested that the 2.4 GHz band to be used for basic internet tasks such as web browsing, email, and downloads, and the 5 GHz band for streaming multimedia. Note that not all 802.11n adapters support dual bands as some only operate at 2.4 GHz.

- Connect to the Home Router using the web browser and click Wireless.

- Click on Basic Wireless Settings. Edit the 2.4 GHz network with the following settings:

Network name: your_first_name (e.g. mahmood) , click Save Settings

Security mode: Click on Wireless Security and select WPA2 Personal for 2.4 GHz

Use Passphrase: csis2270

Leave the others unchanged, and  click Save Settings

- Go back to basic wireless settings, edit the 5 GHz network with Network name: your_first_name5 (e.g. mahmood5), click Save Settings

- Use the smartphone to see if you can connect to the wireless networks.

- Can you connect to the 2.4 GHz network? yes (on the smartphone use mahmood for SSID, select WAP2-PSK and csis 2270 for passphrase)

- Can you connect to the 5 GHz network? yes (on the smartphone use mahmood5 for SSID, Authentication disabled)

D. Router Configuration – Network Security by MAC filtering	[_____/4]

Besides using WPA2, another way to protect your wireless network is by mean of MAC filtering. Every network device has a unique, 12-digit MAC (Media Access Control) address. Using MAC filtering, you can allow only known MAC addresses (known devices) onto your network. You can also exclude specific MAC addresses and deny their access to your wireless network.

- Using ipconfig /all, determine the MAC address (physical address) of the Ethernet adapter of your PC. The MAC address is  00:02:162b:b7:41 (e.g. 00:23:18:fc:7c:87)

- On your Home Router, go to Wireless  Wireless MAC Filter  check Enabled for Wireless MAC Filters

- Select Permit PCs listed below to access wireless network

- Add in the MAC address of your PC (note: separate every 2 digits by a colon “:”).

- Click Save Settings

- Use the smartphone and try to connect to your wireless network. Is it successful? 		No

- Disable the MAC filtering and use the smartphone to try to connect to the network again. Is it successful? Yes

- Find the connected devices list on your home router by clicking Status tab in the top main bar  in the sub bar below it find Local Network and click it  then click on DHCP Client Table

List devices which are connected to the network by writing the MAC and IP of each device

E9-56-E8-9C-99-94    179.200.206.60

E3-CB-4C-32-27-94    250.55.100.29

Lab Submission instructions:

- Save your report file as yourFirstname_yourID_Lab2.docx.
      (example: Nelson_1234_Lab2.docx)

- Save your Packet Tracer file as yourFirstname_yourID_Lab2.pkt 
      (example: Nelson_1234_Lab2.pkt)

- Put both files ( .pkt and .docx) in one folder and name it as   
                        yourFirstname_yourID_Lab2

- Compress the folder into a zip file

- Send the compressed zip file to your instructor not later 6:15 pm of the same day where the lab took place through a Blackboard message strictly (do not send labs by email please. Any lab submitted by email will be ignored).

- Late submissions will not be marked and the student will lose the mark of that lab.

- Students who don’t save lab files with proper names as indicated in 1,2,3 above, will lose 50% of the lab’s mark.

******************************THE END******************************